# Electronics Documentation

### 1. Electronics Block Diagram

### 2. Components Table

| Component | Quantity | Purpose | Justification |
| --- | --- | --- | --- |
| INPUT |  |  |  |
| Pushbuttons | 10 | Physical keypad for OTP entry. | Provides tactile, physical interaction for the user to input the generated code. |
| Ultrasound Sensor [HC-SR04] | 1 | Measures hand distance between the user and the safe [math equation] | Allows for touchless interaction and precise distance measurements (evaluated with a +/- 2.0cm tolerance). |
| PROCESSING |  |  |  |
| ESP32 | 1 | - Receives input (from BLE, pushbuttons and HC-SR04) - Receives and executes system logic - Controls Hardware (servo, stepper, NeoPixel, buzzer) | Required for built-in BLE capabilities to communicate with the app, plus the GPIO pins |
| OUTPUT |  |  |  |
| Servo motor | 1 | Operates the main sliding door mechanism | - Provides exact positional control (110 to 70 degrees) to reliably open and close the main door. - Most preferable as our main door had a linear sliding mechanism (using a scissor structure). - A quick momentum was needed and its small size made it easier to position it below the door. |
| Stepper motor | 1 | Operates the mini locker mechanism | - Wave-drive stepping allows for accurate rotational control. - Preferable as its light (was required to be stuck on the wall). - Anti-clockwise, clockwise rotation. |
| NeoPixel | 1 | Visual feedback and custom color sequences - Correct sequence – green - Wrong sequence – red - Mini locker – multicolour | Requires only one data pin to control multiple LEDs, saving crucial GPIO pins on the ESP32. |
| Buzzer | 1 | Auditory feedback - Different frequency for each pushbutton when OTP is entered. - Correct sequence - Wrong sequence - Opening of main locker and mini locker | Crucial for accessibility and user feedback |
| POWER |  |  |  |
| SMPS Adaptor | 1 | Provides voltage to power module | To convert wall AC power into a stable, high-current 5V DC power source for the entire circuit. |
| Power Module | 1 | - Distributes power to high-draw components - Provides voltage to NeoPixel, servo and stepper motor | The ESP32 cannot safely output enough current to drive the motors and NeoPixel simultaneously; external power prevents brownouts. |
| Motor Driver | 1 | Controls the stepper motor | To adjust the voltage of the coils within the stepper motor. |
| EXTRA |  |  |  |
| Jumper wires (Male and Female) | Many. | Connection of all the components | Makes tighter connections, easier to tape up as compared to normal wire. |
| 10k ohm resistor | 1 | External pull-up resistor for Pin 35 (pushbutton) | Pin 35 is input-only and lacks an internal pull-up resistor; required to prevent floating signals and "ghost typing." |