ODT Summative 2

Idea Brief :

Our Smart Locker System is a multi-gateway security locker that combines physical
hardware with a custom-built mobile application to create a layered, interactive locking mechanism. This system offers three completely distinct ways to authenticate, each one built around a different sensing principle.

| PHYSICAL STRUCTURE | DESCRIPTION |
| --- | --- |
| Main Locker | Primary enclosure with the core locking mechanism, push buttons, NeoPixel, buzzer, servo motor, door sensor, and ultrasonic sensor. |
| Mini Locker | Smaller compartment physically attached beside the main locker, with independent locking mechanism, only accessible after the main locker is opened. |
| Back Wall (main locker) | The interior back wall of the main locker has 4 coloured sheets (physical, non-electronic) fixed to it, revealed only once the main door swings open, serving as the visual clue for the mini locker sequence. |
| Distance Scale | A physical scale marking affixed to the front surface of the locker beside the ultrasonic sensor, so users can position their hand at the correct distance during Gateway 2. |

Gateway 1 – One-time OTP password

CONCEPT

A one-time password is generated each session from the MIT App Inventor application. The user reads the OTP from the app and enters it physically on the locker using push buttons.

How It Works

- The app generates a randomised numeric OTP (within a defined range/digit count) each time the user requests one.

- The OTP is transmitted to the ESP32 over Bluetooth.

- The user enters the code digit-by-digit using physical push buttons mounted on the locker face.

- The ESP32 compares the entered sequence against the received OTP in real time.

### Pushbutton Sound Feedback

Every individual button press triggers the buzzer at a distinct, short frequency tone, giving the user audio confirmation of each digit entered.

| COMPONENT | CORRECT ENTRY RESPONSE | WRONG ENTRY RESPONSE |
| --- | --- | --- |
| NeoPixel | lights up green (sequentially) | lights up red (sequentially) |
| Servo Motor | Rotates to open position [PWM] | -No response- |
| Buzzer | Door sensor detects door physically swinging open, triggers buzzer at frequency 1 | -No door sensor response- |

### Lockout Condition

After 3 consecutive wrong attempts, the entire Gateway 1 input system locks out, push buttons stop responding, no further entries are accepted.

Gateway 2 – Math Equation w/ Ultrasound sensor distance

CONCEPT

Instead of entering a code, the user must solve a math problem and physically express the answer by positioning their hand at the correct distance from an ultrasonic sensor mounted on the locker.

### How It Works

- The app generates a simple arithmetic equation whose answer always falls within a defined range, ideally 5 to 20(e.g. "3 × 4 + 2 = ?", answer 14).

- The equation is displayed on the app interface.

- A physical scale or ruler is affixed to the locker beside the ultrasonic sensor, marked in centimetres.

- The user solves the equation mentally, then places their hand at the corresponding centimetre mark in front of the sensor.

- The ESP32 reads the ultrasonic distance.

- If correct → same success response as Gateway 1 (green NeoPixel, servo opens, door sensor triggers buzzer)

- If wrong → same rejection response (red NeoPixel, rejection buzzer, no servo)

Gateway 3 - Colour Sequence [Mini Locker]

### CONCEPT

### Can only be unlocked after the main locker has already been opened, using a colour sequence revealed on the interior back wall.

How It Works

- Once the main locker door opens (via Gateway 1 or 2), a sequence of 4 coloured NeoPixels illuminates on the back interior wall.

- This sequence is the "key" to the mini locker

- The user opens the MIT App to the mini locker interface

- The interface contains colour-selectable elements that are deliberately not designed to look like obvious buttons (they could be decorative shapes, abstract icons, or ambient UI elements) hiding the interaction in plain sight.

- The user taps the 4 colours in the correct sequence as shown inside the main locker

- ESP32 compares it against the currently active colour sequence

- Correct → mini locker servo opens

- Wrong → rejection response on NeoPixel and buzzer

App Interface

| COMPONENT | FUNCTION |
| --- | --- |
| OTP generation button | Random four digit OTP is generated when button is pressed to be entered into the safe |
| Maths equation generator | Random maths equation is generated, whose answer is the distance in cm at which the user’s hand is to be kept for the Ultrasound sensor to sense it |
| Coloured buttons (4) | When pressed in the right order will unlock the mini locker |
| OTP Display | Displays the OTP generated on pressing the button |
| Equation Display | Displays the equation generated on pressing the button |

How it works

When OTP button is pressed, the code generates a random OTP and sends it to the phone via Bluetooth, where it appears on the display. Entering this OTP using push buttons opens the door.

When pressing the equation generator button, an equation is generated and displayed. When this equation is solved and your hand is placed at that distance, the sensor senses it and the door of the main locker opens.

When the coloured buttons are pressed according to the pattern on the wall inside the safe, the mini locker door opens.

Basic Idea Sketch :