# Test Plan & Results

### 1. Test Plan

| Test | Method | Expected Result |
| --- | --- | --- |
| Gateway 1: OTP Functionality | Generate a 4-digit OTP on the app and input the correct code using the physical pushbutton keypad. | Buzzer plays a success melody, NeoPixel ring turns green, and the main sliding door opens. |
| Gateway 1: Security Lockout | Intentionally input an incorrect 4-digit code three times consecutively. | NeoPixel turns red, buzzer plays a low error tone, and the system enters "Lockout Mode," requiring an app reset. |
| Gateway 2: Math & Distance Logic | Solve the app-generated equation and hold a hand steady at the calculated distance from the ultrasonic sensor. | Sensor identifies the steady hand within a +/−2cm tolerance; NeoPixel turns green and the main door opens. |
| Gateway 2: Human Error Exception | Move hand rapidly in front of the ultrasonic sensor while in Gateway 2 mode. | The "Steady Hand" timer resets; the locker remains closed until the hand is held still for 1.5 seconds. |
| Gateway 3: Pattern Sequence | Input the correct 4-color sequence through the MIT App Inventor interface. | NeoPixel flashes a random color sequence, the buzzer beeps, and the stepper motor rotates to open the mini locker. |
| Mechanical Stability: Servo Speed | Trigger the opening and closing of the main sliding door. | The servo uses a "sweep" function to move at a controlled speed, preventing the door axle from popping off due to torque. |
| BLE Performance: Range & Lag | Send commands (5, 6, or pattern digits) from the mobile app at a distance of 5 meters. | The ESP32 receives commands instantly without disconnection; the console logs "Bluetooth Received" immediately. |
| System Power: Load Management | Trigger a success sequence where NeoPixel, Buzzer, and Stepper Motor are active simultaneously. | The 5V SMPS adapter provides stable current; the ESP32 does not brown out or reboot during peak power draw. |

### 2. User Testing

The users:

- Devoshree [Batchmate]

- Ananya Sreejith [Batchmate]

- Devashri [Batchmate

- Harroop [Faculty]

- Nayan [Faculty]

- Sumit [Faculty]

### 3. Results

| Test | Outcome | Pass/Fail |
| --- | --- | --- |
| Gateway 1: OTP Functionality | Users could generate and enter the code. | Pass |
| Gateway 1: Security Lockout | A few users tried this out when told about this feature. It worked. | Pass |
| Gateway 2: Math & Distance Logic | The user could only navigate through after guidance. Struggled to understand the app interface and could only understand how to position their hands in front of the sensor after we helped them. | Fail |
| Gateway 2: Human Error Exception | Successful | Pass |
| Gateway 3: Pattern Sequence | Users did not get the secret way of opening the mini locker unless told. The stepper motor stopped working. | Fail |
| Mechanical Stability: Servo Speed | Servo was coded right. It managed to only open the main door as it involved pulling. It couldn’t push the door back to its closed position because of mechanism error. | Pass (servo worked) Fail (mechanism-wise) |
| BLE Performance: Range & Lag | Users could try multiple gateways at the same time without getting constantly disconnected (except for when we presented to Nayan and Sumit ) | Pass |
| System Power: Load Management | Stable throughout | Pass |

### 4. Observations

- Users couldn’t navigate through the app interface without our guidance. It seemed unaesthetic and rushed.

- Our batchmates did like the mechanism and the feeling of manually entering the code and placing their hands. They received timely feedback from the locker and liked how interactive it was.

- The scissor mechanism of the main locker door (sliding) kept popping of. The axle just couldn’t handle an excessive amount of attempts. Due to this we had to constantly change the code (slightly), as when we tried to insert the axle back to its place, we changed the start and end position unintentionally.

### 6. Improvements Suggested

- User-friendly and aesthetic app interface.

- A way to avoid frequent disconnection of Bluetooth.

- Re-iterating door mechanism and making it robust so that the model isn’t compromised.

- Make the entire experience of the model more independent in general so that we, as builders, don’t have to interfere their process of interaction. (For example, user couldn’t understand how to place their hands in front of the sensor, a visual signalling how to do it would have helped)